# 4 SAC on Dense Reach: Off-Policy Learning with Maximum Entropy

Vlad Prytula

2026-02-27

Table of Contents

# 4 SAC on Dense Reach: Off-Policy Learning with Maximum Entropy

This chapter covers:

- Why deterministic policies are brittle -- and how the maximum entropy objective keeps exploration alive by rewarding high-entropy action distributions alongside high reward

- Implementing SAC from scratch: replay buffer, twin Q-networks with clipped double Q-learning, squashed Gaussian policy with tanh bounds, automatic temperature tuning, and the full update loop

- Verifying each component with concrete checks (tensor shapes, Q-value ranges, log-probability correctness) before assembling the complete algorithm

- Bridging from-scratch code to Stable Baselines 3 (SB3): confirming that both implementations compute the same squashed Gaussian log-probabilities, and mapping SB3 TensorBoard metrics to the code you wrote

- Training SAC on FetchReachDense-v4 to 100% success rate, matching PPO's performance while establishing the off-policy replay machinery that Chapter 5 (HER) requires

In Chapter 3, you derived the PPO clipped surrogate objective, implemented it from scratch, bridged to SB3, and trained a policy to 100% success on FetchReachDense-v4. The entire pipeline -- environment, network, training loop, evaluation -- is validated and working.

But PPO has a structural limitation that matters for what comes next. PPO is on-policy: every transition is used for a handful of gradient steps, then discarded. For FetchReachDense, where dense rewards provide continuous feedback at every timestep, this wastefulness is tolerable -- PPO still reaches 100% success in about 5 minutes. But each MuJoCo simulation step costs real CPU time. PPO achieves roughly 1,300 steps per second yet uses each frame only once. For harder tasks with sparser signal (coming in Chapter 5), reusing data becomes essential.

This chapter introduces SAC (Soft Actor-Critic), an off-policy algorithm that stores every transition in a replay buffer and reuses it across many updates. SAC adds a maximum entropy bonus that keeps the policy exploratory early in training and lets it become deterministic as it converges. You will derive the maximum entropy objective, implement SAC from scratch (replay buffer, twin Q-networks, squashed Gaussian policy, automatic temperature tuning), verify each component, bridge to SB3, and match PPO's 100% success on FetchReachDense-v4 -- validating the off-policy stack.

One note on why this matters beyond sample efficiency: SAC's replay buffer also enables a technique we will need in Chapter 5. HER (Hindsight Experience Replay) relabels failed transitions with alternative goals -- manufacturing success signal from failure. HER requires off-policy learning because relabeled data did not come from the current policy. The off-policy machinery you build in this chapter is the foundation HER needs.

## 4.1 WHY: The sample efficiency problem

### The standard RL objective (review)

Recall from Chapter 3 that standard RL maximizes expected return:

Jθ=Et=0T−1γtrt

where πθ is the policy with parameters θ, rt is the reward at timestep t, γ∈[0,1) is the discount factor (0.99 in our experiments), and T=50 is the episode horizon. This finds a policy that accumulates high reward. But the optimal policy under this objective is deterministic -- once you know the best action for each state, there is no reason to do anything else.

### Why determinism is a problem

In theory, a deterministic optimal policy is fine. In practice, it causes three problems that matter for robotics.

Problem 1: exploration dies. A deterministic policy exploits what it knows. If the current best action gets reward -0.1, the policy commits to it. But what if there is an action that would get reward -0.01 which the policy has never tried because it stopped exploring? With continuous action spaces (4D in Fetch), the chance of stumbling onto a good action by noise alone is small. The policy gets stuck in a local optimum.

Problem 2: brittleness. A policy that commits fully to one action per state is fragile. Small perturbations -- observation noise, model mismatch between simulation and hardware -- can push it into unfamiliar states where it has no idea what to do. In robotics, sim-to-real transfer regularly exposes this failure mode: a policy that works perfectly in simulation often fails on real hardware.

Problem 3: training instability. When the policy is nearly deterministic, small changes in value estimates cause large behavioral changes (the "winning" action flips). This amplifies noise in the training process and can lead to oscillating or diverging training curves.

### The maximum entropy objective

SAC addresses these problems by modifying the objective. Instead of maximizing reward alone, we maximize reward plus an entropy bonus:

Definition (Maximum entropy objective).

Motivating problem. The standard RL objective produces deterministic policies that stop exploring, are brittle to perturbations, and cause training instability. We need a way to keep the policy stochastic -- preferring a spread of actions -- while still pursuing high reward.

Intuitive description. Instead of asking "which single action is best?", we ask "which distribution over actions gives the best tradeoff between reward and keeping our options open?" The policy prefers actions proportionally to how good they are, rather than committing entirely to the single best one.

Formal definition. The maximum entropy objective augments the standard return with an entropy bonus:

JMaxEntθ=Et=0T−1γtrt+αHπθ⋅∣st

where Hπθ⋅∣st=−Ea∼πθlogπθa∣st is the entropy of the policy at state st -- a measure of how "spread out" the action distribution is. The temperature parameter α>0 controls the tradeoff: higher α favors exploration (high entropy), lower α favors exploitation (high reward). Setting α=0 recovers the standard objective.

Grounding example. Consider a state where two actions have Q-values Qs,a1=−0.1 and Qs,a2=−0.3. Under the standard objective, the policy assigns all probability to a1. Under maximum entropy with α=0.2, the policy assigns probabilities proportional to expQ/α: πa1∝exp−0.1/0.2=exp−0.5≈0.61 and πa2∝exp−0.3/0.2=exp−1.5≈0.22. After normalizing: πa1≈0.73 and πa2≈0.27. The better action is more likely, but the worse action is not eliminated -- the policy retains the ability to discover it was wrong about a1.

Non-example. High entropy does NOT mean random. A policy with α>0 still strongly prefers high-Q actions -- it just does not go all-in on the single best one. As training proceeds and Q-estimates become accurate, α decreases toward zero and the policy becomes nearly deterministic. The entropy bonus is a training aid, not a permanent handicap.

### The Boltzmann policy

The maximum entropy objective has a clean solution. The optimal policy under this objective assigns action probabilities proportional to exponentiated Q-values:

π*a∣s∝expQ*s,a/α

This is a Boltzmann distribution (sometimes called a "softmax" over continuous actions). The temperature α controls the sharpness of the distribution:

- α→0: the distribution collapses to a point mass on the best action (deterministic, like standard RL)

- α→∞: the distribution approaches uniform (every action equally likely)

The Boltzmann form is what makes SAC elegant. Rather than adding exploration noise externally (like epsilon-greedy or Ornstein-Uhlenbeck noise in DDPG), the exploration behavior emerges naturally from the objective itself. The policy explores because exploring is rewarded.

### Off-policy learning: reusing experience

Before we get to SAC's specific components, we need to formalize how it differs from PPO at a structural level.

Definition (Off-policy learning). An algorithm is off-policy if it can learn from data collected by any policy -- including old versions of the current policy, a random policy, or a different agent entirely. Transitions are stored in a replay buffer and reused across many gradient updates. The key property is that the data distribution and the policy being optimized are decoupled.

Compare this to PPO's on-policy constraint (defined in Chapter 3): PPO can only use data from the current policy. Every time the parameters change, old data becomes invalid for computing unbiased gradients, so it must be discarded.

Here is what this means concretely. PPO collects 8,192 transitions, uses them for 10 gradient epochs, then throws them all away. SAC stores every transition in a replay buffer (capacity 1,000,000) and samples from it repeatedly. A transition collected at step 10,000 might be sampled again at step 500,000 -- the same data contributes to learning hundreds of times.

This difference drives the entire chapter. Off-policy learning is:

- More sample-efficient -- every simulation step contributes to many updates, not just one

- More complex -- learning from stale data introduces challenges (overestimation bias, moving targets) that require new machinery (twin critics, target networks)

- Required for HER -- Chapter 5's goal relabeling modifies stored transitions after the fact, which is only possible if transitions are stored and reusable

### Automatic temperature tuning

Choosing α manually requires care. Too low and the policy stops exploring early, settling on a suboptimal solution. Too high and the policy spends time exploring randomly instead of exploiting what it has learned. The right value depends on the task, the training stage, and the action dimensionality.

SAC can learn α automatically by targeting a desired entropy level. The idea: define a target entropy H (how stochastic you want the policy to be), and adjust α to keep the policy's actual entropy near that target.

The target entropy is typically set to −dimA, the negative of the action dimensionality. For Fetch tasks with 4D actions, H=−4. This is a heuristic from Haarnoja et al. (2018b) -- roughly, one nat of entropy per action dimension is enough to maintain useful exploration without being overly random.

The temperature loss function is:

Lα=Ea∼π−αlogπa∣s+H

The gradient pushes α in the right direction:

- If the policy's entropy is below the target (logπ is large and negative, but not negative enough), α increases to encourage more exploration

- If the policy's entropy is above the target, α decreases to allow more exploitation

In practice, SAC learns logα rather than α directly, ensuring α stays positive. We initialize logα=0 (so α=1.0) and let the optimizer adjust it.

### Why this matters for robotics

In robotics, robustness matters as much as performance. A policy needs to handle real sensor noise, not just clean simulation. The maximum entropy objective helps in three specific ways:

- Diverse training data. The policy explores many actions, seeing more of the state space during training. This produces a critic with better coverage -- it has seen more state-action combinations, so its value estimates are more reliable.

- Robust behaviors. By not committing fully to a single action, the policy develops softer trajectories that tolerate perturbations. In our experiments, the entropy bonus at training time translates to less jerky motion even after α has decreased near zero.

- Smoother training. The Boltzmann-style policy changes gradually as Q-values change, avoiding the oscillations that plague deterministic policy optimization.

## 4.2 HOW: The SAC algorithm

### The components

SAC maintains five networks:

| Network | Purpose | Updates |
| --- | --- | --- |
| Actor πθ | Maps states to action distributions (squashed Gaussian) | Policy gradient |
| Critic 1 Qϕ1 | Estimates Q-values | Bellman backup |
| Critic 2 Qϕ2 | Second Q estimate (reduces overestimation) | Bellman backup |
| Target Critic 1 Qϕ1 | Stable target for critic updates | Polyak averaging |
| Target Critic 2 Qϕ2 | Stable target for critic updates | Polyak averaging |

Why two critics? Q-learning tends to overestimate values because we take a maximum over noisy estimates: EmaxQ1,Q2≥maxEQ1,EQ2. Using two critics and taking the minimum for the target counteracts this -- it is called clipped double Q-learning (Fujimoto et al., 2018).

Why target networks? If we update the critic using its own predictions as targets, we get a moving-target problem -- the thing we are trying to match keeps changing. Target networks are slow-moving copies that provide stable optimization targets. They are updated via Polyak averaging (also called soft update):

ϕ←τϕ+1−τϕ

with τ=0.005 (the target network inherits 0.5% of the main network's weights per update). This keeps targets stable while slowly tracking the improving critic.

### The training loop

repeat:
    1. Collect transition using current policy, store in replay buffer
    2. Sample minibatch from replay buffer
    3. Update critics: minimize Bellman error (target uses min of two Q-targets)
    4. Update actor: maximize Q-value + entropy
    5. Update temperature alpha (if auto-tuning)
    6. Soft-update target networks (Polyak averaging)

Step 1: Collect data. Unlike PPO, which collects full trajectories before updating, SAC collects one transition at a time and updates after each step. The transition s,a,r,s′,done goes into the replay buffer.

Step 2: Sample minibatch. Uniformly sample batch_size transitions from the buffer. This is where data reuse happens -- the same transition might be sampled many times across training.

Step 3: Update critics. For each critic, minimize the squared Bellman error against a soft target (detailed in Section 4.6).

Step 4: Update actor. Maximize expected Q-value plus entropy bonus (detailed in Section 4.7).

Steps 5-6: Temperature and target updates. Adjust α to maintain target entropy, then blend the main critic weights into the targets.

### PPO versus SAC

Both algorithms solved FetchReachDense-v4 in Chapter 3 and this chapter respectively. Here is how they compare:

| Aspect | PPO (Chapter 3) | SAC (this chapter) |
| --- | --- | --- |
| Data reuse | None (on-policy: use once, discard) | Extensive (replay buffer, reuse many times) |
| Exploration | Optional entropy bonus (we used 0.0) | Core: entropy is in the objective |
| Sample efficiency | Low (discards data) | High (reuses data) |
| Stability | High (clipped updates bound change) | Medium (moving targets, overestimation risk) |
| Complexity | Lower (1 actor + 1 critic) | Higher (1 actor + 2 critics + 2 targets + alpha) |
| Wall-clock speed | Faster per step (~1,300 fps) | Slower per step (~594 fps) |

The throughput difference comes from network count: SAC updates five networks per step versus PPO's one shared network. But SAC compensates by extracting more learning from each transition. The tradeoff is clear: PPO is simpler and faster per step; SAC is more complex but more efficient per sample. For the tasks ahead -- especially sparse rewards with HER in Chapter 5 -- sample efficiency wins.

### Key hyperparameters

| Parameter | Default | What it controls |
| --- | --- | --- |
| buffer_size | 1,000,000 | Replay buffer capacity (how far back we remember) |
| batch_size | 256 | Minibatch size for each gradient update |
| learning_starts | 10,000 | Steps of random data collection before training begins |
| tau | 0.005 | Target network update rate (Polyak averaging) |
| ent_coef | "auto" | Entropy temperature (auto-tuned by default) |
| learning_rate | 3e-4 | Gradient step size for all optimizers |
| gamma | 0.99 | Discount factor |

For FetchReachDense-v4, SB3 defaults work well. We recommend training with these values first and only tuning if something goes wrong (see What Can Go Wrong later in this chapter).

Figure 4.1 shows the full SAC architecture -- how data flows from the environment through the replay buffer and into the three update steps.

SAC architecture diagram showing the data flow: environment transitions flow into a circular replay buffer, minibatches are sampled to update the twin Q-networks (critic loss), the actor (policy loss), and the temperature alpha, with target networks updated via Polyak averaging

Figure 4.1: The SAC architecture. Transitions flow from the environment into a circular replay buffer. At each update step, a minibatch is sampled and used to update three components: the twin Q-networks (minimizing Bellman error against slow-moving target networks), the actor (maximizing Q-value plus entropy), and the temperature α (maintaining target entropy). Target networks track the main critics via Polyak averaging (τ=0.005). (Illustrative diagram.)

## 4.3 Build It: Replay buffer

Off-policy learning requires storing transitions for reuse. Unlike PPO, which discards data after each update, SAC stores every transition in a circular buffer and samples from it repeatedly.

A replay buffer is a fixed-size array. New transitions overwrite the oldest when the buffer is full:

[t_0, t_1, t_2, ..., t_{n-1}, t_n, t_{n+1}, ...]
 ^-- oldest                          ^-- newest

With a capacity of 1,000,000 and 1M training steps, each transition is stored once but sampled many times. The buffer size controls the "memory horizon" -- how far back the agent remembers. Too small and you lose useful old experience; too large and you dilute recent (better) experience with outdated data.

Each entry stores a five-tuple: s,a,r,s′,done -- the state, action, reward, next state, and termination flag. Sampling is uniform random: every transition in the buffer has an equal chance of being drawn into a minibatch.

# Replay buffer for off-policy learning
# (from scripts/labs/sac_from_scratch.py:replay_buffer)

class ReplayBuffer:
    def __init__(self, obs_dim, act_dim, capacity=100000):
        self.capacity, self.ptr, self.size = capacity, 0, 0
        self.obs = np.zeros((capacity, obs_dim), dtype=np.float32)
        self.actions = np.zeros((capacity, act_dim), dtype=np.float32)
        self.rewards = np.zeros(capacity, dtype=np.float32)
        self.next_obs = np.zeros((capacity, obs_dim), dtype=np.float32)
        self.dones = np.zeros(capacity, dtype=np.float32)

    def add(self, obs, action, reward, next_obs, done):
        self.obs[self.ptr] = obs
        self.actions[self.ptr] = action
        self.rewards[self.ptr] = reward
        self.next_obs[self.ptr] = next_obs
        self.dones[self.ptr] = done
        self.ptr = (self.ptr + 1) % self.capacity
        self.size = min(self.size + 1, self.capacity)

    def sample(self, batch_size, device):
        idx = np.random.randint(0, self.size, size=batch_size)
        to_t = lambda x: torch.from_numpy(x).to(device)
        return {"obs": to_t(self.obs[idx]),
                "actions": to_t(self.actions[idx]),
                "rewards": to_t(self.rewards[idx]),
                "next_obs": to_t(self.next_obs[idx]),
                "dones": to_t(self.dones[idx])}

The buffer pre-allocates numpy arrays at initialization -- this avoids the overhead of growing a Python list. The ptr (pointer) tracks where the next transition goes, wrapping around via modular arithmetic. Sampling converts numpy arrays to PyTorch tensors on the target device.

Checkpoint. Add 100 transitions with obs_dim=10, act_dim=4 and sample a batch of 32. You should see: obs shape (32, 10), actions shape (32, 4), rewards shape (32,), buf.size == 100, buf.ptr == 100. If shapes are wrong, check that the numpy arrays were initialized with the correct dimensions.

Note on the MLP helper. The TwinQNetwork and GaussianPolicy classes use an MLP helper -- a standard feedforward network with ReLU activations. It lives in the lab file (scripts/labs/sac_from_scratch.py) alongside the snippet regions and takes three arguments: input_dim, output_dim, and hidden_dims (defaulting to [256, 256]). If you are copying code from the chapter, you will need this class. Run --verify to confirm everything assembles correctly.

## 4.4 Build It: Twin Q-network

SAC uses two Q-networks to reduce overestimation bias (as discussed in Section 4.2). Each Q-network takes a state-action pair and outputs a scalar Q-value. Having two independent networks and taking their minimum for the target counteracts the systematic upward bias of Q-learning:

Qϕ1s,a, Qϕ2s,a

The architecture is straightforward -- each Q-network is an MLP that concatenates state and action as input:

# Twin Q-networks for reducing overestimation bias
# (from scripts/labs/sac_from_scratch.py:twin_q_network)

class TwinQNetwork(nn.Module):
    """Twin Q-networks for reducing overestimation."""

    def __init__(self, obs_dim: int, act_dim: int,
                 hidden_dims: list[int] = [256, 256]):
        super().__init__()
        self.q1 = MLP(obs_dim + act_dim, 1, hidden_dims)
        self.q2 = MLP(obs_dim + act_dim, 1, hidden_dims)

    def forward(self, obs, action):
        x = torch.cat([obs, action], dim=-1)
        return (self.q1(x).squeeze(-1),
                self.q2(x).squeeze(-1))

The two networks share no parameters -- they are fully independent MLPs with the same architecture (two hidden layers of 256 units, ReLU activations). The input dimension is obs_dim + act_dim because we concatenate state and action. The output is squeezed from (batch, 1) to (batch,) for convenient loss computation.

Why 256 hidden units? This is the SAC default from Haarnoja et al. (2018a), and it works well for low-dimensional state spaces like Fetch. PPO in Chapter 3 used 64 -- SAC's critics need more capacity because they approximate a function of both state AND action, which has a higher-dimensional input.

Checkpoint. Create a TwinQNetwork(obs_dim=10, act_dim=4) and run a forward pass with batch size 32. You should see: Q1 shape (32,), Q2 shape (32,), both means near 0 at initialization (random weights produce centered outputs), and all values finite. If you get a shape error, check that obs_dim + act_dim matches the MLP input dimension.

## 4.5 Build It: Squashed Gaussian policy

SAC's policy outputs a continuous action distribution. The challenge: Fetch actions must be bounded in −1,1, but a Gaussian distribution has unbounded support. The solution is a squashed Gaussian -- sample from a Gaussian, then apply tanh to bound the result:

z∼Nμθs,σθs2,  a=tanhz

where μθ and σθ are neural network outputs (state-dependent mean and standard deviation).

The complication is in the log-probability. Because tanh is a nonlinear transformation, we need a Jacobian correction to get the correct density. The change-of-variables formula gives:

logπa∣s=logpz∣s−i=1dlog1−tanh2zi

where pz∣s is the Gaussian density and d is the action dimension. The correction term accounts for how tanh compresses the probability density near the boundaries ±1. In code, we use a numerically stable formulation:

# Squashed Gaussian policy
# (from scripts/labs/sac_from_scratch.py:gaussian_policy)

class GaussianPolicy(nn.Module):
    LOG_STD_MIN, LOG_STD_MAX = -20, 2

    def __init__(self, obs_dim, act_dim,
                 hidden_dims=[256, 256]):
        super().__init__()
        self.backbone = MLP(obs_dim, hidden_dims[-1],
                            hidden_dims[:-1])
        self.mean_head = nn.Linear(hidden_dims[-1], act_dim)
        self.log_std_head = nn.Linear(hidden_dims[-1], act_dim)

    def forward(self, obs):
        features = F.relu(self.backbone.net[:-1](obs))
        features = self.backbone.net[-1](features)
        mean = self.mean_head(features)
        log_std = self.log_std_head(features)
        log_std = torch.clamp(log_std,
                              self.LOG_STD_MIN, self.LOG_STD_MAX)
        std = log_std.exp()
        # Reparameterization trick: sample = mean + std * noise
        dist = Normal(mean, std)
        x_t = dist.rsample()                # gradient flows through
        action = torch.tanh(x_t)            # squash to [-1, 1]
        # Log prob with numerically stable squashing correction
        log_prob = dist.log_prob(x_t).sum(dim=-1)
        log_prob -= (2 * (np.log(2) - x_t
                     - F.softplus(-2 * x_t))).sum(dim=-1)
        return action, log_prob

A few things to notice:

- State-dependent std. Unlike PPO in Chapter 3 (which used a state-independent log_std parameter), SAC learns a different standard deviation for each state. This allows the policy to be more exploratory in unfamiliar states and more precise in well-understood ones.

- Clamped log-std. The LOG_STD_MIN = -20 and LOG_STD_MAX = 2 bounds prevent numerical instability. Without them, log_std can go to −∞ (zero variance, division by zero) or +∞ (infinite variance, useless actions).

- Reparameterization trick. We use rsample() instead of sample() so that gradients flow through the sampling operation. This is essential -- without it, we cannot backpropagate through the actor loss.

- Numerically stable log-prob. The squashing correction 2 * (log(2) - x_t - softplus(-2 * x_t)) is mathematically equivalent to log(1 - tanh^2(x_t)) but avoids the numerical issue where tanh^2(x_t) rounds to exactly 1.0 for large x_t, producing log(0) = -inf.

| Math | Code | Meaning |
| --- | --- | --- |
| z∼Nμ,σ2 | x_t = dist.rsample() | Sample from Gaussian (with gradient) |
| a=tanhz | action = torch.tanh(x_t) | Squash to −1,1 |
| logpz | dist.log_prob(x_t).sum(-1) | Gaussian log density |
| −∑log1−tanh2zi | -(2 * (log(2) - x_t - softplus(-2*x_t))).sum(-1) | Jacobian correction |

Checkpoint. Create a GaussianPolicy(obs_dim=10, act_dim=4) and run a forward pass with batch size 32. You should see: actions bounded in −1,1, log-probs finite and negative (probabilities are less than 1, so log-probs are negative), shapes (32, 4) for actions and (32,) for log-probs. If actions exceed −1,1, check that torch.tanh is applied. If log-probs contain NaN or −∞, check the LOG_STD_MIN clamp.

## 4.6 Build It: Twin Q-network loss -- the soft Bellman backup

The critic update minimizes the squared difference between each Q-network's prediction and a target that incorporates the entropy bonus. This is the soft Bellman backup:

Lϕi=Es,a,r,s′∼BQϕis,a−y2

where B is the replay buffer and the Bellman target y is:

y=r+γ1−dminj=1,2Qϕjs′,a′−αlogπa′∣s′

with a′∼π⋅∣s′ sampled from the current policy. The target has three parts: the immediate reward r, the discounted future value (estimated by the minimum of two target Q-networks), and the entropy bonus −αlogπ. The 1−d term zeroes the bootstrap when the episode terminates.

# Twin Q-network loss with soft Bellman backup
# (from scripts/labs/sac_from_scratch.py:twin_q_loss)

def compute_q_loss(q_network, target_q_network, policy,
                   batch, gamma=0.99, alpha=0.2):
    obs, actions = batch["obs"], batch["actions"]
    rewards, next_obs = batch["rewards"], batch["next_obs"]
    dones = batch["dones"]

    q1, q2 = q_network(obs, actions)  # current Q-values

    with torch.no_grad():  # no gradient through targets
        next_actions, next_log_probs = policy(next_obs)
        target_q1, target_q2 = target_q_network(next_obs, next_actions)
        target_q = torch.min(target_q1, target_q2)  # pessimistic
        # Soft Bellman backup
        target = rewards + gamma * (1.0 - dones) * (
            target_q - alpha * next_log_probs)

    q1_loss = F.mse_loss(q1, target)
    q2_loss = F.mse_loss(q2, target)

    info = {"q1_loss": q1_loss.item(),
            "q2_loss": q2_loss.item(),
            "q1_mean": q1.mean().item(),
            "q2_mean": q2.mean().item(),
            "target_q_mean": target.mean().item()}
    return q1_loss + q2_loss, info

The torch.no_grad() block is critical -- we do not backpropagate through the target computation. The target networks and the policy provide fixed targets for this update step. If gradients flowed through the target, the optimization would chase a moving target, defeating the purpose of having separate target networks.

| Math | Code | Meaning |
| --- | --- | --- |
| Qϕis,a | q1, q2 | Current Q-value predictions |
| minjQϕjs′,a′ | torch.min(target_q1, target_q2) | Pessimistic target (reduces overestimation) |
| −αlogπa′∣s′ | -alpha * next_log_probs | Entropy bonus in target |
| y | target | The Bellman target |
| 1−d | (1.0 - dones) | Zero bootstrap at episode end |

Checkpoint. Create a TwinQNetwork, copy it to make a target_q_network, create a GaussianPolicy, and compute the loss on a random batch. You should see: q1_loss finite and typically below 1.0 at initialization (random networks with zero-centered targets produce small errors), q1_mean near 0, target_q_mean near 0. If the loss is very large (above 100), check that rewards are bounded and that the done mask is applied correctly.

## 4.7 Build It: Actor loss with entropy

The policy update maximizes Q-values while maintaining entropy. The actor loss is:

Lθ=Es∼B, a∼πθαlogπθa∣s−mini=1,2Qϕis,a

Read this as: "find actions that have high Q-values (−Q is minimized) while keeping the policy spread out (αlogπ penalizes low entropy)." The balance between these two objectives is controlled by α.

# Actor loss with entropy regularization
# (from scripts/labs/sac_from_scratch.py:actor_loss)

def compute_actor_loss(
    policy, q_network, obs, alpha=0.2,
):
    # Sample actions from current policy
    actions, log_probs = policy(obs)

    # Evaluate with Q-networks (use min for pessimism)
    q1, q2 = q_network(obs, actions)
    q_value = torch.min(q1, q2)

    # Loss = E[alpha * log_pi - Q]  (minimize)
    actor_loss = (alpha * log_probs - q_value).mean()

    info = {
        "actor_loss": actor_loss.item(),
        "entropy": -log_probs.mean().item(),  # H = -E[log pi]
        "log_prob_mean": log_probs.mean().item(),
    }
    return actor_loss, info

Notice that actions are sampled fresh from the current policy (not taken from the replay buffer). The Q-networks evaluate these new actions using their current weights. This is different from the critic update, which evaluated the actions that were actually taken when the transition was collected. The actor asks: "given this state, what action would I take NOW, and how good would it be?"

The entropy diagnostic H = -E[log pi] tracks how stochastic the policy is. At initialization, entropy is high (the policy has not learned any preferences). As training proceeds and α decreases, entropy drops and the policy becomes more deterministic. If entropy drops to near zero too early, that is worth investigating -- see What Can Go Wrong for diagnostics.

Checkpoint. Compute the actor loss on random observations with alpha=0.2. You should see: actor_loss finite and positive at initialization (the policy has not learned to select high-Q actions yet), entropy positive (the random-initialized policy is stochastic), and log_prob_mean negative (probabilities are less than 1). If actor_loss is NaN, check that the squashed Gaussian log-prob computation in Section 4.5 is numerically stable.

## 4.8 Build It: Automatic temperature tuning

The actor loss in Section 4.7 uses a fixed α=0.2. That works for a single verification step, but in real training the right value is hard to choose ahead of time. Too low and exploration dies early. Too high and the policy wastes time on random actions. The right α depends on the task, the training stage, and the action dimensionality -- it changes as training progresses.

SAC solves this by learning α alongside the policy and critics. The idea (introduced in Section 4.1): define a target entropy H=−dimA -- for Fetch's 4D action space, H=−4 -- and adjust α to keep the policy's actual entropy near that target.

The temperature loss function is:

Lα=Ea∼π−αlogπa∣s+H

where logπa∣s comes from the current policy's output (the same log-probabilities we computed in Section 4.5). The gradient pushes α in the right direction: if entropy is below the target, α increases to encourage more exploration; if entropy is above the target, α decreases to allow more exploitation.

In practice, we optimize logα rather than α directly. Since exp⋅ is always positive, this ensures α>0 without needing a constrained optimizer.

# Automatic temperature tuning
# (from scripts/labs/sac_from_scratch.py:temperature_loss)

def compute_temperature_loss(
    log_alpha,         # learnable log(alpha)
    log_probs,         # log pi(a|s) from policy
    target_entropy,    # target entropy (-dim(A))
):
    alpha = log_alpha.exp()

    # If log_pi > -target (entropy too low): loss positive, alpha increases
    # If log_pi < -target (entropy too high): loss negative, alpha decreases
    alpha_loss = -(alpha * (log_probs.detach()
                            + target_entropy)).mean()

    info = {"alpha_loss": alpha_loss.item(),
            "alpha": alpha.item()}
    return alpha_loss, info

Notice the .detach() on log_probs -- we do not want the temperature gradient to flow back through the policy network. The temperature update adjusts only α, taking the current policy's entropy as a given. The policy has its own optimizer (Section 4.7) that handles its parameters.

| Math | Code | Meaning |
| --- | --- | --- |
| α | log_alpha.exp() | Temperature (always positive) |
| logπa∣s | log_probs | Policy log-probabilities (detached) |
| H=−dimA | target_entropy | Target entropy (-4 for Fetch) |

Checkpoint. Initialize log_alpha = torch.tensor(0.0, requires_grad=True) so that α starts at 1.0. After a few optimizer steps on random log-probs, α should have changed from 1.0 -- the direction depends on whether the random policy's entropy is above or below the target. The key check is that α remains positive (it always will, since we optimize logα). If alpha_loss is NaN, check that log_probs contains finite values.

Figure 4.2 shows what automatic temperature tuning looks like over a full training run. Early in training, when the policy is uncertain and entropy is naturally high, α is relatively large (around 0.47 at 30k steps). As the policy learns and becomes more deterministic, α drops -- reaching 0.0004 by 1M steps. The policy transitions from exploratory to exploitative without any manual schedule.

Entropy coefficient alpha over training steps, starting around 0.47 and decreasing to near 0.0004 over 1M steps, with annotations showing exploration phase (high alpha) and exploitation phase (low alpha)

Figure 4.2: Entropy coefficient α over training on FetchReachDense-v4. The automatic temperature tuning starts α around 0.47 (exploration phase) and drives it to 0.0004 (exploitation phase) as the policy converges. This happens without any manual schedule -- the dual gradient descent adjusts α to maintain the target entropy of H=−4. (Generated from TensorBoard logs in runs/sac/FetchReachDense-v4/seed0/, metric replay/ent_coef.)

## 4.9 Build It: SAC update -- wiring it together

We now have six components: replay buffer, twin Q-networks, squashed Gaussian policy, critic loss, actor loss, and temperature loss. The SAC update function wires them into a single gradient step. The sequence matters:

- Update Q-networks: minimize Bellman error (uses current policy for next-action sampling)

- Update policy: maximize Q-value plus entropy (uses updated Q-networks)

- Update temperature: maintain target entropy (uses updated policy's log-probs)

- Soft-update target networks: Polyak averaging ϕ←τϕ+1−τϕ

The first listing shows the three gradient updates -- Q-networks, policy, and temperature -- each using the loss functions we built in Sections 4.6-4.8:

# SAC update: gradient steps
# (from scripts/labs/sac_from_scratch.py:sac_update)

def sac_update(
    policy, q_network, target_q_network, log_alpha,
    policy_optimizer, q_optimizer, alpha_optimizer,
    batch, target_entropy, gamma=0.99, tau=0.005,
):
    alpha = log_alpha.exp().item()

    # 1. Q-network update
    q_loss, q_info = compute_q_loss(
        q_network, target_q_network, policy,
        batch, gamma, alpha)
    q_optimizer.zero_grad()
    q_loss.backward()
    q_optimizer.step()

    # 2. Policy update
    actor_loss, actor_info = compute_actor_loss(
        policy, q_network, batch["obs"], alpha)
    policy_optimizer.zero_grad()
    actor_loss.backward()
    policy_optimizer.step()

    # 3. Temperature update
    with torch.no_grad():
        _, log_probs = policy(batch["obs"])
    alpha_loss, alpha_info = compute_temperature_loss(
        log_alpha, log_probs, target_entropy)
    alpha_optimizer.zero_grad()
    alpha_loss.backward()
    alpha_optimizer.step()

Ordering matters. The Q-networks update first because the actor uses Q-values to evaluate its actions. The temperature updates last because it needs the updated policy's log-probs. Each component has its own Adam optimizer with learning rate 3e-4 -- coupling them into one optimizer would mix gradients that serve different objectives. We re-run the policy under torch.no_grad() to get fresh log-probs for the temperature update, because the policy parameters changed in step 2.

After the three gradient steps, the target networks blend in the new critic weights via Polyak averaging:

# 4. Target network update (Polyak averaging)
    with torch.no_grad():
        for p, tp in zip(q_network.parameters(),
                         target_q_network.parameters()):
            tp.data.copy_(tau * p.data + (1 - tau) * tp.data)

    return {**q_info, **actor_info, **alpha_info}

This is a direct weighted blend of parameters, not a gradient step. It runs in torch.no_grad() and takes negligible time. The target network inherits 0.5% of the main network's weights per update (τ=0.005), providing the stable optimization targets that the critic loss (Section 4.6) depends on.

Checkpoint. Initialize all networks, create three Adam optimizers, and run 20 updates on a random batch. You should see: alpha has changed from 1.0 (it may go up or down depending on the random policy's entropy relative to the target), q1_loss is finite, actor_loss is finite, and all values in the info dictionary are finite. If any value is NaN after 20 updates, check that each optimizer is attached to the correct parameters and that no gradients flow where they should not (target computation, temperature log-probs).

### Quick verification: all seven components at once

Before running the full demo, verify that all seven components assemble and produce sane values:

bash docker/dev.sh python scripts/labs/sac_from_scratch.py --verify

Expected output:

============================================================
SAC From Scratch -- Verification
============================================================
Q-network:  Q1 mean ~0.04, Q2 mean ~0.06 (near zero at init)
Policy:     actions in [-0.99, 0.97], log_prob mean -2.63
SAC update: Q1 loss 2.35, actor loss -4.19, alpha 1.00 -> 0.99
            All values finite after 20 updates.
[PASS] All checks passed.
============================================================

This runs in under 30 seconds on CPU. If any value is NaN or if shapes are wrong, go back to the component that failed -- the error messages point to the specific check.

### The demo: SAC solves Pendulum from scratch

The wired-up implementation can solve a real continuous control task. Run (~5 minutes on CPU, ~2 minutes on GPU):

bash docker/dev.sh python scripts/labs/sac_from_scratch.py --demo --steps 50000

Here are the results we got (your numbers may vary slightly with different seeds):

| Step | Avg Return | Alpha | What's happening |
| --- | --- | --- | --- |
| 5,000 | -843 | 0.415 | Learning started, still mostly random |
| 10,000 | -190 | 0.197 | Solved (threshold: -200) |
| 25,000 | -141 | 0.048 | Refining swing-up behavior |
| 50,000 | -134 | 0.017 | Stable, nearly deterministic policy |

The policy solves Pendulum by step 10,000 -- well within 50k. The temperature drops from 0.415 to 0.017, showing the automatic tuning at work: early exploration gives way to late exploitation without any manual schedule. Figure 4.3 shows the learning curve.

This is the same algorithm SB3 runs. The difference is pedagogical clarity, not algorithmic. Every gradient step you see in SB3's TensorBoard traces back to the functions you just built.

Learning curve from the SAC from-scratch demo on Pendulum-v1: average return on the y-axis improves from around -1200 to above -200 over 50k training steps on the x-axis, with a dashed line at -200 marking the solved threshold

Figure 4.3: Learning curve from the from-scratch SAC implementation on Pendulum-v1. The average return improves from random (~-1200) to solved (above -200) within 10k steps, then refines further to ~-134 by 50k steps. The dashed line marks the -200 solved threshold. This validates that all seven components -- replay buffer, twin Q-networks, squashed Gaussian policy, critic loss, actor loss, temperature tuning, and the wiring -- are correct. (Generated by python scripts/labs/sac_from_scratch.py --demo --steps 50000.)

## 4.10 Bridge: From-scratch to SB3

We have built SAC from scratch and verified each component. SB3 implements the same math in a production library. Before we use SB3 for the real training run, let's confirm that the two implementations agree on the core computation.

The key quantity to compare is the squashed Gaussian log-probability -- the mathematical object that appears in the actor loss (Section 4.7), the critic target (Section 4.6), and the temperature loss (Section 4.8). If the log-probs match, the losses match, and the gradients point in the same direction.

The bridging proof feeds the same random observations and pre-squash samples through our GaussianPolicy.forward() and through SB3's SquashedDiagGaussianDistribution. Both apply tanh squashing and compute log-probabilities, but with slightly different numerical formulas:

- Our implementation: log_prob = dist.log_prob(x_t) - 2*(log(2) - x_t - softplus(-2*x_t))

- SB3's implementation: log_prob = dist.log_prob(x_t) - log(1 - tanh(x_t)^2 + 1e-6)

The formulas are mathematically equivalent for the squashing correction. The ~0.02 nat typical difference comes from SB3's 1e-6 epsilon safety term, which prevents log(0) near the boundaries but introduces a small bias.

Run the comparison:

bash docker/dev.sh python scripts/labs/sac_from_scratch.py --compare-sb3

Expected output:

============================================================
SAC From Scratch -- SB3 Comparison
============================================================
Max abs log_prob diff: 2.055e-02
Tolerance (atol):      5.0e-02

[PASS] Our squashed Gaussian log_prob matches SB3

The match is within tolerance (0.02 < 0.05). For non-saturated actions -- the vast majority during training -- both formulas agree much more closely. The epsilon matters only when tanhu→1, which happens at the action boundaries.

### What SB3 adds beyond our from-scratch code

Our implementation handles one environment and stores transitions in a simple numpy buffer. SB3 adds engineering features that matter for real training:

- Vectorized environments. n_envs=1 (SAC default in SB3) with SubprocVecEnv or DummyVecEnv. Unlike PPO which benefits heavily from parallel environments, SAC's off-policy nature means a single environment can fill the replay buffer adequately.

- Efficient replay buffer. SB3's replay buffer handles dictionary observations natively (the observation, achieved_goal, and desired_goal keys from Fetch), stores them efficiently in numpy arrays, and supports batched sampling.

- MultiInputPolicy. SB3 builds separate encoders for each key in the dictionary observation and concatenates them. Our from-scratch code assumed a flat observation vector.

- Gradient clipping. SB3 clips gradient norms by default, preventing a single bad batch from causing an explosively large update.

- Automatic entropy coefficient with constrained optimization -- the same math as our compute_temperature_loss, but with additional numerical safeguards.

### Mapping SB3 TensorBoard metrics to our code

When you train with SB3 and open TensorBoard, the logged metrics correspond directly to the functions you built:

| SB3 TensorBoard key | Our function | What it measures |
| --- | --- | --- |
| train/actor_loss | compute_actor_loss | Policy loss (entropy-regularized) |
| train/critic_loss | compute_q_loss | Twin Q-network Bellman error |
| replay/ent_coef | log_alpha.exp() | Temperature α (auto-tuned) |
| replay/q1_mean | TwinQNetwork Q1 output | Average Q1 value in sampled batch |
| replay/q2_mean | TwinQNetwork Q2 output | Average Q2 value in sampled batch |
| replay/q_min_mean | torch.min(q1, q2) | Pessimistic Q-value estimate |
| rollout/ep_rew_mean | (environment) | Mean episode return |
| rollout/success_rate | (environment) | Fraction of episodes where goal is reached |

We find this mapping useful for debugging. When you see replay/ent_coef = 0.08 in TensorBoard, that is α=0.08 -- the output of the same log_alpha.exp() from Section 4.8. When you see train/critic_loss = 0.34, that is the MSE Bellman error from compute_q_loss in Section 4.6. Having built each component yourself, you can trace any TensorBoard metric back to the exact computation that produced it.

## 4.11 Run It: Training SAC on FetchReachDense-v4

A note on script naming. The production script is called ch03_sac_dense_reach.py because the repository's tutorial numbering differs from the Manning chapter numbering. In the tutorials, SAC on dense Reach is chapter 3; in this book, it is chapter 4. The script name and artifact paths (checkpoints, eval JSONs, TensorBoard directories) all use ch03. This is intentional -- renaming would break the tutorial infrastructure. When you see ch03 in a command or file path, you are running the right script for this chapter.

---------------------------------------------------------
EXPERIMENT CARD: SAC on FetchReachDense-v4
---------------------------------------------------------
Algorithm:    SAC (soft actor-critic, off-policy, auto-tuned entropy)
Environment:  FetchReachDense-v4
Fast path:    500,000 steps, seed 0
Time:         ~14 min (GPU) / ~60 min (CPU)

Run command (fast path):
  bash docker/dev.sh python scripts/ch03_sac_dense_reach.py all \
    --seed 0 --total-steps 500000

Checkpoint track (skip training):
  checkpoints/sac_FetchReachDense-v4_seed0.zip

Expected artifacts:
  checkpoints/sac_FetchReachDense-v4_seed0.zip
  checkpoints/sac_FetchReachDense-v4_seed0.meta.json
  results/ch03_sac_fetchreachdense-v4_seed0_eval.json
  runs/sac/FetchReachDense-v4/seed0/    (TensorBoard logs)

Success criteria (fast path):
  success_rate >= 0.95
  mean_return > -5.0
  final_distance_mean < 0.03

Full multi-seed results: see REPRODUCE IT at end of chapter.
---------------------------------------------------------

### Running the experiment

The one-command version:

bash docker/dev.sh python scripts/ch03_sac_dense_reach.py all --seed 0

This runs training, evaluation, and comparison to PPO. It takes about 14 minutes on a GPU. For a quick sanity check that finishes in about 2 minutes:

bash docker/dev.sh python scripts/ch03_sac_dense_reach.py train --total-steps 100000

If you are using the checkpoint track (no training), the pretrained checkpoint is at checkpoints/sac_FetchReachDense-v4_seed0.zip. You can evaluate it directly:

bash docker/dev.sh python scripts/ch03_sac_dense_reach.py eval \
  --ckpt checkpoints/sac_FetchReachDense-v4_seed0.zip

### Training milestones

Watch for these milestones during training. Note that learning_starts=10000 means no training happens during the first 10k steps -- the agent collects random data to seed the replay buffer:

| Timesteps | Success Rate | What's happening |
| --- | --- | --- |
| 0-10k | 0% | Collecting random data (no training yet) |
| 10k-50k | 0-5% | Training begins, Q-values adjusting |
| 50k-150k | 20-60% | Rapid improvement, entropy decreasing |
| 150k-400k | 80-99% | Policy converging |
| 400k-1M | 100% | Fine-tuning, entropy near zero |

Our test run achieved 100% success rate after approximately 300k steps, with an average goal distance of 18.6mm (the success threshold is 50mm) and throughput of approximately 594 steps per second on an NVIDIA GB10.

### Reading TensorBoard

Launch TensorBoard to watch training in real time:

bash docker/dev.sh tensorboard --logdir runs --bind_all

Then open http://localhost:6006 in your browser. Here is what healthy training looks like:

| Metric | Expected behavior | What to watch for |
| --- | --- | --- |
| rollout/ep_rew_mean | Steadily increasing (less negative) | Should move from around -20 toward -1 |
| rollout/success_rate | 0 -> 1 over training | The primary success metric |
| replay/ent_coef | Starts high (~0.47), decreases gradually | Should NOT drop to near-zero in the first 10k steps |
| replay/q_min_mean | Stabilizes in a reasonable range | Should NOT grow unbounded above 100 |
| train/actor_loss | Fluctuates, generally decreases | NaN means numerical instability |
| train/critic_loss | Starts high, decreases and stabilizes | Tracks learning progress |

These are the same quantities from the Build It sections: replay/ent_coef maps to log_alpha.exp(), train/critic_loss maps to compute_q_loss, and replay/q_min_mean maps to torch.min(q1, q2). The TensorBoard metric mapping table in Section 4.10 has the full correspondence.

Tip. Low GPU utilization (~5-10%) is expected. The bottleneck is CPU-bound MuJoCo simulation, not neural network operations. With small networks (two 256-unit hidden layers) and batch sizes (256), GPU operations complete in microseconds while the CPU runs physics. This is normal for state-based RL -- pixel-based policies in later chapters will use the GPU much more heavily.

### SAC versus PPO: results comparison

Both algorithms solve FetchReachDense-v4, but with different tradeoffs. Figure 4.4 compares their learning curves.

| Metric | PPO (Chapter 3) | SAC (this chapter) |
| --- | --- | --- |
| Success Rate | 100% | 100% |
| Mean Return | -0.40 | -1.06 |
| Final Distance | 4.6mm | 18.6mm |
| Action Smoothness | 1.40 | 1.68 |
| Training Time | ~6 min | ~28 min |
| Throughput | ~1,300 fps | ~594 fps |

Both achieve 100% success -- the off-policy stack is validated. SAC has a higher final distance (18.6mm vs 4.6mm) but still well within the 50mm threshold. SAC is slower in wall-clock time because it updates more networks per step (actor + two critics + two targets versus PPO's single shared network). SAC's slightly rougher actions (smoothness 1.68 vs 1.40) reflect the entropy bonus encouraging a spread of actions during training.

Both algorithms solve this task completely, so the interesting comparison is structural: PPO used each transition once and discarded it, while SAC stored every transition and reused it across many updates. On FetchReachDense, where signal is plentiful, this difference is a mild efficiency trade. On sparse-reward tasks (Chapter 5), the replay buffer becomes the key enabler for learning.

Learning curve comparison: PPO and SAC success rate over timesteps on FetchReachDense-v4, both reaching 100% but PPO reaching it somewhat earlier in wall-clock time while SAC has a different convergence profile

Figure 4.4: Learning curve comparison -- PPO (Chapter 3) versus SAC (this chapter) on FetchReachDense-v4. Both reach 100% success rate. PPO converges somewhat earlier measured by timesteps because its per-step updates are lighter. SAC converges to a stable solution with the replay buffer machinery that Chapter 5 requires. (Generated by extracting rollout/success_rate from TensorBoard logs in runs/ppo/FetchReachDense-v4/seed0/ and runs/sac/FetchReachDense-v4/seed0/.)

### Verifying results

After training completes, check the evaluation JSON:

cat results/ch03_sac_fetchreachdense-v4_seed0_eval.json | python -m json.tool | head -20

Key fields to verify:

{
  "aggregate": {
    "success_rate": 1.0,
    "return_mean": -1.06,
    "final_distance_mean": 0.019
  }
}

The passing criteria are: success rate above 95%, mean return above -5.0, and final distance below 0.03 meters. Our runs consistently exceed these thresholds. If yours do not, see What Can Go Wrong below.

## 4.12 What can go wrong

Here are the failure modes we have encountered, organized by symptom. For each, we give the likely cause and a specific diagnostic.

### replay/q_min_mean grows unbounded (above 100 and still rising)

Likely cause. Overestimation feedback loop: Q-targets use overestimated Q-values, which train the critic to overestimate further, which produces even higher targets. The twin Q-network minimum (Section 4.4) is supposed to prevent this, but it can still happen if target networks update too fast or rewards are misconfigured.

Diagnostic. Check three things: (1) rewards are bounded -- FetchReachDense should produce rewards in −1,0; (2) target networks update with small τ (0.005, not 0.5 or 1.0) -- verify the Polyak averaging is actually running; (3) try reducing the learning rate from 3e-4 to 1e-4 to slow down the overestimation spiral.

### replay/ent_coef drops to less than 0.01 within the first 10k steps

Likely cause. The policy collapsed to near-deterministic before learning anything useful. This can happen if the target entropy is too low or if the initial policy variance is too small.

Diagnostic. Check the target entropy setting -- it should be −dimA=−4 for Fetch. Try temporarily using a fixed entropy coefficient (ent_coef=0.2 in SB3) to isolate whether the issue is auto-tuning or something else. If the fixed coefficient works, the auto-tuning's initial learning rate may be too aggressive -- try reducing the alpha learning rate to 1e-4.

### Success rate stalls below 50% for more than 200k steps

Likely cause. Insufficient exploration or replay buffer sampling issues.

Diagnostic. Check the entropy coefficient -- if it is very low (below 0.01) early in training, the policy may have stopped exploring. Verify that learning_starts is not set too high (default 10,000 is appropriate). Check that batch_size is reasonable (256) -- too small means high-variance updates, too large means slow updates.

### Training much slower than expected (below 200 fps on GPU)

Likely cause. GPU not in use, or excessive gradient steps per environment step.

Diagnostic. Check nvidia-smi inside the container -- a python process should be using GPU memory. Verify gradient_steps=1 (the default, meaning one gradient step per environment step). Note that approximately 594 fps is typical for SAC on FetchReach with a GPU -- the bottleneck is CPU-bound MuJoCo simulation, not the neural networks.

### Q1 and Q2 diverge significantly (difference greater than 10x)

Likely cause. One Q-network is stuck or has a different effective learning rate.

Diagnostic. Verify that both Q-networks share the same optimizer -- our compute_q_loss returns q1_loss + q2_loss and both should receive gradients through a single q_optimizer.step(). If you accidentally created separate optimizers for Q1 and Q2, check that both are being stepped.

### ep_rew_mean flatlines near -20 for the entire run

Likely cause. Wrong environment or the policy is not receiving goal information.

Diagnostic. Check the env_id is FetchReachDense-v4 (not the sparse variant FetchReach-v4, which produces rewards of exactly 0 or -1). Verify that SB3 is using MultiInputPolicy -- this handles the dictionary observation structure. Print obs.keys() to confirm you see observation, achieved_goal, and desired_goal.

### --compare-sb3 shows log-prob difference above 0.05

Likely cause. The squashing correction formula differs in a way that matters.

Diagnostic. Verify that your implementation uses the numerically stable formula: 2*(log(2) - x_t - softplus(-2*x_t)). This is mathematically equivalent to log(1 - tanh^2(x_t)) but avoids log(0) at the boundaries. If the difference is between 0.02 and 0.05, it is likely harmless -- the discrepancy comes from SB3's epsilon term and only affects near-saturated actions.

### Build It --verify reports NaN in Q-loss or actor loss

Likely cause. Numerical instability in the squashed Gaussian log-probability computation.

Diagnostic. Check the LOG_STD_MIN and LOG_STD_MAX bounds in GaussianPolicy. Without them, the log standard deviation can go to −∞ (producing zero variance and division by zero) or +∞ (producing infinite variance and meaningless actions). The safe range is −20,2. Also verify that the softplus-based squashing correction is used instead of the naive log(1 - tanh^2(x)), which produces −∞ when tanh(x) rounds to exactly ±1.

## 4.13 Summary

This chapter derived SAC from the maximum entropy objective and built it from the ground up. Here is what you accomplished:

- The maximum entropy objective. Instead of maximizing reward alone, SAC maximizes reward plus an entropy bonus: JMaxEnt=Et​γtrt+αHπ. This keeps the policy exploratory during training and lets it become deterministic as it converges. The optimal policy assigns probabilities proportional to exponentiated Q-values -- a Boltzmann distribution.

- Off-policy learning. SAC stores every transition in a replay buffer and reuses it across many updates, unlike PPO which discards data after each update. This makes SAC more sample-efficient and -- crucially -- enables the goal relabeling that Chapter 5 requires.

- From-scratch implementation. You built seven components: the replay buffer, twin Q-networks, squashed Gaussian policy, twin Q-network loss (soft Bellman backup), actor loss with entropy, automatic temperature tuning, and the full update step. Each was verified with concrete checks before wiring them together.

- Bridge to SB3. The bridging proof confirmed that our squashed Gaussian log-probability matches SB3's distribution implementation within 0.02 nats. SB3 adds engineering features (efficient replay storage, dictionary observation handling, gradient clipping) but computes the same underlying math.

- Pipeline validation. SAC achieved 100% success rate on FetchReachDense-v4, matching PPO's performance. The entire off-policy stack -- replay buffer, twin critics, automatic temperature tuning, Polyak averaging -- is validated and working.

That last point is the foundation for what comes next. Chapter 5 introduces Hindsight Experience Replay (HER), which tackles sparse rewards -- first on Reach (where the improvement is marginal) and then on Push (where it is transformative, lifting success from 5% to 99%). With sparse rewards, the agent receives a signal of either 0 (success) or -1 (failure) -- no distance information, no gradient toward the goal. Almost every episode is a failure, and failures carry no information about how to improve.

HER solves this by asking a simple question after each failed episode: "I did not reach the desired goal, but what goal DID I reach?" It then relabels the transitions in the replay buffer with the achieved goal, turning a failure into a (synthetic) success. This goal relabeling requires two things that you now have: (1) an off-policy algorithm that can learn from modified transitions, and (2) a replay buffer where those transitions are stored and accessible for relabeling. The off-policy machinery you built in this chapter is the foundation HER needs.

## Reproduce It

---------------------------------------------------------
REPRODUCE IT
---------------------------------------------------------
The results and pretrained checkpoints in this chapter
come from these runs:

  for seed in 0 1 2; do
    bash docker/dev.sh python scripts/ch03_sac_dense_reach.py all \
      --seed $seed --total-steps 1000000
  done

Hardware:     Any machine with Docker (GPU optional; tested on NVIDIA GB10)
Time:         ~28 min per seed (Linux GPU), ~120 min per seed (Mac/CPU)
Seeds:        0, 1, 2

Artifacts produced:
  checkpoints/sac_FetchReachDense-v4_seed{0,1,2}.zip
  checkpoints/sac_FetchReachDense-v4_seed{0,1,2}.meta.json
  results/ch03_sac_fetchreachdense-v4_seed{0,1,2}_eval.json
  runs/sac/FetchReachDense-v4/seed{0,1,2}/

Results summary (what we got):
  success_rate: 1.00 +/- 0.00  (3 seeds x 100 episodes)
  return_mean:  -0.93 +/- 0.13
  final_distance_mean: 0.016 +/- 0.003

If your numbers differ by more than ~10%, check the
"What Can Go Wrong" section above.

The pretrained checkpoints are available in the book's
companion repository for readers using the checkpoint track.
---------------------------------------------------------

## Exercises

1. (Verify) Reproduce the single-seed baseline.

Run the fast path command and verify your results match:

bash docker/dev.sh python scripts/ch03_sac_dense_reach.py all \
  --seed 0 --total-steps 500000

Check the eval JSON: success_rate should be >= 0.95, mean_return > -5.0. Record your training time and steps per second for comparison with Chapter 3 (PPO). Expected: approximately 594 fps (versus PPO's approximately 1,300 fps), due to more network updates per step.

2. (Tweak) Twin Q-network ablation.

In the lab's compute_q_loss(), the target uses torch.min(target_q1, target_q2). Change it to use only one Q-network: target_q = target_q1. Run --verify and observe: does q1_mean grow larger without the min clipping? Run --demo --steps 50000 and compare final returns. Expected: without the min trick, Q-values may overestimate, leading to slightly worse or unstable training.

3. (Tweak) Fixed versus auto-tuned temperature.

In the lab's sac_update(), replace alpha = log_alpha.exp().item() with a fixed value: alpha = 0.2. Run --demo --steps 50000 with fixed versus auto-tuned. Compare: (a) final return, (b) convergence speed, (c) policy entropy at the end. Expected: auto-tuned reaches better final performance because it reduces α as the policy improves; fixed α=0.2 may over-explore late in training.

4. (Extend) Add Q-value divergence monitoring.

In the verification code, add a check that tracks |q1_mean - q2_mean| over updates. Plot this divergence over the 20 verification steps. Expected: the two Q-networks should stay close (within approximately 0.5 of each other) because they see the same targets. Large divergence would indicate a problem -- for instance, if only one Q-network receives gradient updates.

5. (Challenge) SAC on Pendulum with different target entropies.

Modify the demo to try target entropy values of -0.5, -1.0 (default for 1D), and -2.0. For each, train for 50k steps and record: (a) final average return, (b) final alpha value, (c) steps to reach -200 return. Expected: lower target entropy leads to earlier exploitation (faster convergence but potentially less robust). Higher target entropy leads to more exploration (slower convergence but the policy may discover better strategies). This trade-off is the heart of the maximum entropy objective.